18 Sep update — SUMO pipeline and first M1 results

Jaeyun Ree · 18 September 2026 · for discussion with Eunus (Aamir optional). Follows the 7 Sep report on the exact optimum.

# 1. What we did

- Built the SUMO pipeline end to end on a real corridor: M1 Monash Freeway inbound, Springvale Rd → CityLink, 20.5 km, 31 road segments, 16 ramps (8 on, 8 off).

- Wrote a converter that turns SUMO output into the paper's vehicle format, so greedy and ILA run on it unchanged.

- Measured how the real corridor differs from the uniform synthetic data used in the paper.

- Ran greedy and ILA on the M1 instance over AV:PV ratios 0.2–0.8 (Eunus's question on the ratio).

- Surveyed Victorian open traffic-count data (DTP) to see whether demand can be calibrated to real counts.

# 2. How

OpenStreetMap → netconvert → randomTrips (placeholder demand, 1 hour, 1800 vehicles) → sumo → converter → greedy / ILA.

- Positions are projected onto the paper's 0–100 grid: 1 unit = 205 m, so L_min = 10 units ≈ 2 km.

- Each vehicle's speed and entry time are fitted from its simulated trajectory (constant-speed fit, as the model assumes).

- AV / PV labels are assigned at random (20 % AV for the smoke run; varied for the ratio sweep). SUMO has no such concept; the rule is ours to decide.

- Time tolerance τ = 5 model time units, as in the paper. One model time unit on the M1 is about 8 s (see §4).

Caveat. Demand is a randomTrips placeholder: only the road network is real. Where vehicles enter and exit, and how long they stay, is shaped by the real ramps; how many take each route is still uniform. Structure numbers below hold regardless; saving percentages do not.

Figure 1. The M1 inbound corridor as the algorithms see it: 16 ramps on the 100-unit grid, lane count, and a 3.6 km stretch with no junction.

# 3. Results

## 3.1 What the real corridor changes (independent of the demand model)

|  | Synthetic (paper) | M1 inbound |
| --- | --- | --- |
| Possible entry points | 101 (uniform) | 9 (corridor start + 8 on-ramps) |
| Possible exit points | 101 (uniform) | 9 (8 off-ramps + corridor end) |
| Origin–destination pairs that occur | thousands | 44 |
| Trips shorter than L_min (≈ 2 km) | 0 by design | 18 % (adjacent ramps are 0.6 km apart in places) |
| Speeds | 0.8–1.2 (arbitrary) | mean 88 km/h, range 15–120, from car-following |

Figure 2. Entry position, exit position and trip length: uniform synthetic data (left) vs the M1 corridor (right). Dashed lines are ramp positions.

## 3.2 The constant-speed assumption breaks on long trips

The model assumes each vehicle keeps one speed. Fitting the best constant speed to each simulated trajectory and measuring the residual gives a lower bound on that error:

| Trip length | Vehicles | Median max error | Share above τ = 5 s |
| --- | --- | --- | --- |
| < 4 km | 710 | 0.6 s | 0 % |
| 4–10 km | 435 | 2.5 s | 36 % |
| 10–15 km | 322 | 13.9 s | 95 % |
| > 15 km | 282 | 21.9 s | 100 % |

Lane count changes (3 → 6), merges and exit slow-downs accumulate over 20 km. If τ = 5 s is read literally, the model is outside its tolerance for almost every trip longer than 10 km. This connects to Revision Plan item 10 (temporal synchronisation).

## 3.3 Greedy vs ILA on the M1, by AV:PV ratio

Five seeds of random AV / PV labels on placeholder demand — a first look, not a result. Mean ± standard deviation across seeds.

| AV : PV | 0.2 | 0.4 | 0.6 | 0.8 |
| --- | --- | --- | --- | --- |
| ILA − greedy, time constraints OFF | 0.00 | 0.00 | 0.00 | 0.00 |
| ILA − greedy, time constraints ON | +0.77 ± 0.33 pp | +1.40 ± 0.40 pp | +1.08 ± 0.20 pp | +0.41 ± 0.17 pp |
| Saving, ILA, time ON | 45.1 % | 61.1 % | 59.5 % | 54.7 % |
| PV distance covered | 53 % | 85 % | 96 % | 98 % |

Figure 3. ILA − greedy against AV:PV ratio (left) and saving level (right): synthetic data (blue, 4–8 seeds per point) vs M1 (red, 5 seeds, mean ± sd).

- Without time constraints the two algorithms tie exactly at every ratio. ILA's advantage is temporal — the same finding as on synthetic data and in the 7 Sep report.

- With time constraints the gap is an inverted U, peaking at ratio 0.4 on the M1 (0.8 on synthetic), consistently across all five seeds. By ratio 0.6, 96 % of PV distance is already towed, so there is nothing left to compete for.

- The paper's saving metric divides by AV + PV distance, so it falls past 0.4 as AV distance (which can never be saved) grows in the denominator. PV coverage rises monotonically.

## 3.4 Reminder from the 7 Sep report

Against the exact optimum (21 instances up to 15 AV / 30 PV) greedy reaches 93.3 % and ILA 93.1 %. The shortfall is choice quality, not model expressiveness: restricting the model to the segments the algorithms can produce costs only 0.65 %.

# 4. What this means

- SUMO did not widen the ILA–greedy gap. +0.4 to +1.4 pp (5-seed means) on the M1 versus +0.2 to +1.3 pp on synthetic data. Realistic structure does not turn a 1-pp difference into a 5-pp one. Three independent checks (synthetic re-analysis, exact optimum, SUMO) now agree.

- The 7 % headroom to the optimum is seven times the ILA–greedy difference. Both heuristics under-use the multi-segment hand-off that the exact solutions exploit (more, shorter segments; 88 % of PVs served vs 75 %).

- The paper never fixes what τ = 5 s means physically. On the M1 one model time unit is about 8 s, so τ = 5 is either 5 s (saving halves: 24 %) or ≈ 42 s (50–52 %). Both readings are self-consistent; the paper has to pick one.

- Real demand data exists but covers 40 % of the corridor. DTP TIRTL detectors (15-min counts, CC BY 4.0) sit at 16 main-line sites between Springvale Rd and Warrigal Rd plus the High St ramp; nothing from Burke Rd to CityLink (toll road). All are point counts, no origin–destination information.

# 5. Feedback I need

- The claim. Given 1–2 above, the Revision Plan's Phase-2 decision point ("if the gap does not widen, the contribution needs rethinking") has arrived early. Options: (A) a method that closes part of the 7 % — I would test a local-search post-processor for two weeks against the 21 exact instances; (B) reposition the paper as problem + exact benchmark + realistic evaluation, with ILA as "greedy-quality, faster"; (C) 2D — but the exact-solver diagnosis says the loss is choice quality, not expressiveness, so 2D alone would not change it. My proposal: B as the floor, A as a two-week test. Do you agree with the reading and the test?

- AV:PV ratio. Your note "specialised → quite high": did you mean the ratio should be high (many AVs) or the gap should be high? My framing: ratio = (share of heavy vehicles that can tow) ÷ (share of cars opting in). What range should the sweep cover, and which metric — the paper's saving over AV + PV distance, or PV distance covered?

- AV / PV labelling in SUMO. Random fraction (current placeholder), by vehicle class (trucks / buses = AV), or something else?

- Demand. (i) keep the placeholder; (ii) partial calibration to TIRTL counts via routeSampler — 2–3 days, gives "calibrated to DTP detector counts" for the eastern 40 %, must be labelled partial; (iii) full calibration — not possible with open data. I recommend (ii) for this paper and (iii) for the 2D paper.

- Physical unit of τ. 5 real seconds or 5 model units (≈ 42 s)? Under (ii) entry times come from real-second counts, so real seconds is the natural reading. Should long trips use piecewise speeds given the 22 s drift?

- Scale. Keep 400 PV / 100 AV as the headline, or a full peak hour (≈ 1750 vehicles on the main line)?

# 6. Proposed next steps (pending the above)

- If (A) is approved: local-search post-processor, scored against the 21 exact instances; report at the next fortnightly update.

- If (ii) is approved: download one TIRTL month, map detectors to edges, enumerate candidate routes, routeSampler, report GEH.

- Implement the agreed labelling rule and τ convention; re-run the ratio sweep with 10+ seeds and confidence intervals.

- Extract 5-minute sub-windows of the M1 scenario (≈ 10 AV / 25 PV) so the exact solver can score both heuristics on real structure.

Code and notes: github.com/JayReeMarine/… — sumo/NOTES-sumo.md (SUMO track), NOTES.md (exact solver), README.md "Revision Work" for the commands.